NIT SCHEDULE

Short term maintenance of Takhatpur Ex. Road from Km. 26.542 to Km. 32.093, Mungeli Ex. Road from Km. 45.786 to Km. 56.275, Pandariya Ex. Road from Km. 82.582 to Km. 87.133 and Pondi Ex. Road from Km. 103.166 to Km. 104.756 = 22.181 Km. on N.H.-130A in the  state of Chhattisgarh

PAC Rs. 226.87 Lacs (Including GST)

| S.No. / ItNo. | Item of work |  | Quantity |  |  |  | Rate |  | Unit | Amount |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Road Maintenance Bill No.1 |  |  |  |  |  |  |  |  |  |  |
| 1 10.25 Road SOR | Dressing of side shoulder/Scrapping the road side shoulder by Mechanical means (By JCB/DOZER) including excavation of raised side shoulders upto 15cm depth including remooving/cutting bushes, grass and dressing shoulder to desired camber and disposing of excavated stuff up to road boundary etc complete. as directed by Engineer In Charge. |  | 52677.00 |  |  | 5.00 |  |  | Sqm | 263385.00 |  |
|  |  |  |  |  |  | Total |  |  |  | 263385.00 |  |
|  |  | ROAD PROPERTY MAINTENANCE (BILLNo.2) |  |  |  |  |  |  |  |  |  |  |
| 2 8.13 Road SOR |  | Road Marking with Hot Applied Thermoplastic Compound with Reflectorising Glass Bads on Bituminous Surface (Providing and laying of hot applied thermoplastic compound 2.5 mm thick including reflectorising glass beads @ 250 gms per sqm area, thickness of 2.5 mm is exclusive of surface applied glass beads as per IRC:35. The finished surface to be level, uniform and free from streaks and holes. |  | 998.145 | 713.00 |  |  | Sqm. |  |  | 711677.00 |  |
| 3 8.4 Road SOR |  | Retro- reflectrorized Traffic signs (Retro- reflectrorized Traffic signs (Providing and fixing of retro- reflectrorized cautionary, mandatory and informatory sign as per IRC:67-2012 made of retro-reflective type sheeting vide clause 801.3, fixed over aluminum sheeting of required thickness supported on a mild steel frame of size 40 x 40 x 5 mm and mild steel angle iron post 75 mm x75 mm x 6mm firmly fixed to the ground by means of properly designed foundation with M15 grade cement concrete 45 cm x 45cm x 60cm, 60 cm below ground level as per approved drawing and painting all components of the signs and supports as per clause 7.2 of IRC:67-2012) |  |  |  |  |  |  |  |  |  |  |
|  |  | Made of Class-C Micro Prismatic Grade Sheeting of Type IX (for National Highways and State Highways) |  |  |  |  |  |  |  |  |  |  |
|  |  | With Aluminum Sheeting of 2 mm thickness |  |  |  |  |  |  |  |  |  |  |
|  |  | Made of Class-C Micro Prismatic Grade Sheeting of Type-XI (for National Highways and State Highways) |  |  |  |  |  |  |  |  |  |  |
|  |  | (iii) 75 cm equilateral triangle |  | 20 | 5142.00 |  |  | Each |  |  | 102840.00 |  |
|  |  | (xviii) 50 cm x 60 cm rectangular (single chevron with double plate in singe post) |  | 20 | 9204.00 |  |  | Each |  |  | 184080.00 |  |
|  |  | (x) 80 mm x 60 mm rectangular |  | 16 | 7837.00 |  |  | Each |  |  | 125392.00 |  |
|  |  | (xii) 60 cm x 45 cm rectangular |  | 12 | 5444.00 |  |  | Each |  |  | 65328.00 |  |
|  |  | (xv) 90 cm high octagon |  | 16 | 10014.00 |  |  | Each |  |  | 160224.00 |  |
|  |  | (vi) 90 cm circular |  | 10 | 9616.00 |  |  | Each |  |  | 96160.00 |  |
| 4 8.50 Road SOR |  | Providing & Fixing Delineator made of M.S. Hollow Rectangular Pipe (Conforming to IS: 4923) of cross section 100 mm x 80 mm, thickness 4mm, of 1.10 meter length. The top of the hollow pipe, welded and closed with 16 gauge M.S. Sheet. The height of the Delineator above ground level should be between 0.8 meter and 0.9 meter and 0.20 to 0.30 m below ground. The bottom of the delineator should be provided with holdfast of length 15cm on each side. Delineator should be painted with coat of Zinc Chromate stoving Prime and two coats of Black Stove Enamel (Oven Baked) paint with 3nos. 15 cm bands of white Retro-reflective Sheeting of Engineering Grade pasted around the pipe and 2 Nos. High intensity Grade Retro-reflective Sheeting of size 8 cm x 10cm pasted on the top of each side of Delineator and treated With HLVA & fixing the same in M-15 grade concrete block of size 45x43x30cms including transportation etc. complete. The construction and placement of the delineator must beas per IRC: 79. |  | 400 | 1913.00 |  |  | Each |  |  | 765200.00 |  |
| 5 8.51 Road SOR |  | Providing & Fixing Hazard Marker Type-2 Partial Retro Reflective Size 0.30 x 0.90M made of 16 gauge (1.6 mm) thick mild steel sheet Confirming to IS:2062 Painted with one cast of Zinc Chromate Stoving Primer and two coats of Black Stove Enamel (Oven Baked) paint with 10 cm bands of yellow Retro- Reflective Sheeting of Engineering Grade pasted a per IRC:79 including one MS angle Iron post of size 50x50x5 mm of 1.65 meter long duly painted with synthetic enameled paint block including bolts and fixing the same in M-15 concrete black of size 30x30x30cm, including transportation etc. Complete. |  | 22 | 1491.00 |  |  | Each |  |  | 32802.00 |  |
| 6 8.54 Road SOR |  | 8.54 Providing & Fixing tree studs in Square/ Round shape of 4” side/diameter made of 2 mm aluminum substrate (Confirming to IS:736 -Material Designation, 24345 or 1900) end reflective surface of High Intensity grade as specified in section 801.3 of MORT&H duly treated with HLVA and fixing the same with four Nos. high strength nails confirming to 1S:1367 on trees at site each. |  | 200 | 67.00 |  |  | Each |  |  | 13400.00 |  |
| 7 8.05 Bridge SOR |  | Direction and Place Identification signs upto 0.9 sqm size board. (Providing and erecting direction and place identification retro-reflectorised sign as per IRC :67 made of retro reflective type sheeting and vide clause 801.3, fixed over aluminium sheeting, 2 mm thick or Aluminum composite material sheet with overall thickness of 4mm with area not exceeding 0.9 sqm and fixed over back support frame of min 35 x 35 x 3mm Angle mounted on a mild steel circular pipe 65 NB, firmly fixed to the ground by means of properly designed foundation with M15 grade cement concrete 45 x 45 x 60 cm, 60 cm below ground level as per approved drawing) |  |  |  |  |  |  |  |  |  |  |
|  |  | A Made of Class-C Micro Prismatic Grade Sheeting of Type-XI (for National Highways and State Highways |  | 9.00 | 14901.00 |  |  | Sqm. |  |  | 134109.00 |  |
|  |  |  |  | SUB TOTAL |  |  |  |  |  |  | 2391212.00 |  |

PERFORMANCE BASED MAINTENANCE AND PAVEMENT REPAIRS - Bill No.-3

| Item No | Description of Items | Length in Km | Unit | Rate per km per month (in Rs.) | Amount (in Rs.) |
| --- | --- | --- | --- | --- | --- |
| 3.1 | Providing treatment and repair to pot holes/ruts of any depth/patch repair of all types of bitumen pavement including providing seal for filling cracks, WMM, bituminous surfacing, complete as per Technical Specification Clause 3004.2, and as per direction of Engineer by using paving grade bitumen/emulsion/cold ready mix material. The job includes removal of all failed material, trimming of completed excavation to provide firm vertical faces, cleaning of surface, painting of tack coat on the sides and base of excavation as per Clause no. 503, back filling the patch with bituminous material as per Clause no. 509. Compacting trimming and finishing the surface to form a smooth continuous surface all as per Clause no. 3004.2 of Technical Specifications. Carrying out cleaning, removing of dust/silt/thrash from carriageway and median along with cleaning of sign boards, delineators, studs with detergent. | 22.181 | 12 months | 60758.00 | 16172078.00 |
| 3.2 | Repair of localized failed areas of PQC by marking the area, removing the failed area using jack hammer, saw cutting to full depth of PQC, replacing membrane, clearing the area from dust and debris. Placing M-40 grade PQC in a depth equivalent to the existing depth of PQC and installing existing expansion joint/other joint. Repair of transverse joints of cement concrete pavement involving removal of old joint sealent and sealing with fresh sealent (Poly sulphide) including repair of spalled/broken edges of joints with Epoxy mortar complete as per Clause no. 3005 of Technical Specifications | - | - | - | - |
| 3.3 | Maintenance of cuts and fills slopes/shoulders conforming to Technical Specifications Clause no. 3002. Fill material should be brought from borrow areas located outside ROW conforming to Technical Specification no. 305. Watered and well compacted with plate compactor or power rammer with all leads and lifts as directed by the Engineer. | - | - | - | - |
| 3.4 | Removal of rank vegetation/ weeds and undesirable vegetation from shoulders, embankment slopes upto ROW (both sides) complete in all respects including breaking of clods, rough dressing and disposal of waste material and vegetation at a place outside ROW as per direction of Engineer- ln-Charge. | - | - | - | - |
| 3.5 | (a) Removal of dead animals/ birds and burying them at suitable location within two hours. (b) Removal of stray animals to suitable place out of carriageway | 22.181 | 12 Months | 1500.00 | 399258.00 |
|  | TOTAL FOR BILL NO. 3 | Total |  |  | 16571336.00 |
|  |  | Sub Total (Bill No. 01, 02 & 03) |  |  | 1,92,25,933.00 |
|  |  | ADD FOR GST 18% |  |  | 34,60,668.00 |
|  |  | GRAND TOTAL |  |  | 2,26,86,601.00 |
|  |  | Say Rs. |  |  | 226.87 Lakhs |

| Sub Engineer | Sub Divisional Officer P.W.D. N.H. Sub Division Bilaspur (C.G.) | Executive Engineer P.W.D. N.H. Division Bilaspur (C.G.) |
| --- | --- | --- |